- A spectrum analyzer measures the magnitude of an input signal versus frequency within the full frequency range of the instrument. The primary use is to measure the power of the spectrum of known and unknown signals.

- in statistics and signal processing, an algorithm that estimates the strength of different frequency components (the power spectrum) of a time-domain signal. This may also be called frequency domain analysis

- Spectrum analyzer, a hardware device that measures the magnitude of an input signal versus frequency within the full frequency range of the instrument

- While it provides valuable insight into a signal’s properties, signal analysis is only as good as the instrument with which it is performed.

- Its primary use is to measure the power of the spectrum of known and unknown signals.

- In the frequency domain, complex signals (e.g., comprising more than one frequency) are separated into their frequency components, and the level at each frequency is displayed. Frequency-domain measurements have several distinct advantages. For one, information not discernible on an oscilloscope becomes readily apparent on a spectrum analyzer.

In time series analysis, Bartlett's method:

- (also known as the method of averaged periodograms[1]), is used for estimating power spectra. It provides a way to reduce the variance of the periodogram in exchange for a reduction of resolution, compared to standard periodograms.[2][3] A final estimate of the spectrum at a given frequency is obtained by averaging the estimates from the periodograms (at the same frequency) derived from a non-overlapping portions of the original series.

- The method is used in physics, engineering, and applied mathematics. Common applications of Bartlett's method are frequency response measurements and general spectrum analysis.

- The method is named after M. S. Bartlett who first proposed it.[2][3]

## Definition and procedure

Bartlett’s method consists of the following steps:

- The original N point data segment is split up into K (non-overlapping) data segments, each of length M

- For each segment, compute the periodogram by computing the discrete Fourier transform (DFT version which does not divide by M), then computing the squared magnitude of the result and dividing this by M.

- Average the result of the periodograms above for the K data segments.

  - The averaging reduces the variance, compared to the original N point data segment.

The end result is an array of power measurements vs. frequency "bin".

# Welch's method

In physics, engineering, and applied mathematics, Welch's method, named after P.D. Welch, is used for estimating thepower of a signal at different frequencies: that is, it is an approach to spectral density estimation. The method is based on the concept of using periodogram spectrum estimates, which are the result of converting a signal from the time domain to the frequency domain. Welch's method is an improvement on the standard periodogram spectrum estimating method and on Bartlett's method, in that it reduces noise in the estimated power spectra in exchange for reducing the frequency resolution. Due to the noise caused by imperfect and finite data, the noise reduction from Welch's method is often desired.

## Definition and Procedure[edit]

The Welch method is based on Bartlett's method and differs in two ways:

- The signal is split up into overlapping segments: The original data segment is split up into L data segments of length M, overlapping by D points.

  - If D = M / 2, the overlap is said to be 50%

  - If D = 0, the overlap is said to be 0%. This is the same situation as in the Bartlett's method.

- The overlapping segments are then windowed: After the data is split up into overlapping segments, the individual L data segments have a window applied to them (in the time domain).

  - Most window functions afford more influence to the data at the center of the set than to data at the edges, which represents a loss of information. To mitigate that loss, the individual data sets are commonly overlapped in time (as in the above step).

  - The windowing of the segments is what makes the Welch method a "modified" periodogram.

After doing the above, the periodogram is calculated by computing the discrete Fourier transform, and then computing the squared magnitude of the result. The individual periodograms are then time-averaged, which reduces the variance of the individual power measurements. The end result is an array of power measurements vs. frequency "bin".